Hands-on session – UPC tutorial

Supercomputing 2003

11/12/03

UPC Lab 2

2D Heat conduction problem:

- Mathematical background

For heat conduction, the fundamental 2D Partial Differential Equation (1) in a stationary medium is:

∂2T∂x2+∂2T∂y2=1α∂T∂t

where T is temperature, t is time, α is the thermal diffusivity.

We assumed that the source does not generate heat and the thermal properties are constant. In addition, we also assumed a solid surface with α = 1.0.

(1): Modeling of Laser Machining Process, chapter 3.5, Combined Research and Curriculum Development Nontraditional Manufacturing (NTM), Columbia University

(http://www.columbia.edu/cu/mechanical/mrl/ntm/pgIndex.html)

- Finite Difference Method overview

A finite difference method is used to do the numerical approximation of the heat conduction over a solid plate. Thus, a computational grid is mapped onto the surface.

N

N

Solid

The grid has a total dimension of (N+2)*(N+2). The temperatures at the boundaries are fixed at initialization time.

So, for an interior point of the 2D grid, the temperature is calculated as follows:

Ti,jt+1=14Ti−1,jt+Ti+1,jt+Ti,j−1t+Ti,j+1t

Illustration of this stencil operation:

- The sequential C program

The following sequential program (heat_seq.c) is present in the ~/heat_conduction directory.  Please browse through it.

The program heats one of the sides as the initial condition, and iterates to study how the heat transfers across the surface.

Note: This sequential code is not fully optimized, since it is performing a copy of the complete grid[][] into new_grid[][] at the end of each iteration. This could have been avoided using:

  - a pointer flipping (That will be considered in the following exercises, section 5)

or

  - using conditions to alternate between two buffers and decide which is the input buffer and output buffer in a given iteration

This will be addressed in one of the excercies.

//heat_seq.c

#include <stdio.h>

#include <math.h>

#include <sys/time.h>

#define N 30

double grid[N+2][N+2], new_grid[N+2][N+2];

void initialize(void)

{

int j;

/* Heat one side of the solid */

for( j=1; j<N+1; j++ )

{

grid[0][j] = new_grid[0][j] = 1.0;

}

}

int main(void)

{

struct timeval ts_st, ts_end;

double dTmax, dT, epsilon, time;

int finished, i, j, k, l;

double T;

int nr_iter;

initialize();  /*set initial conditions*/

/* Set the precision wanted */

epsilon  = 0.0001;

finished = 0;

nr_iter = 0;

/* Start timing */

gettimeofday( &ts_st, NULL );

do

{

dTmax = 0.0;

for( i=1; i<N+1; i++ )

{

for( j=1; j<N+1; j++ )

{

T = 0.25 *

(grid[i+1][j] + grid[i-1][j] +

grid[i][j-1] + grid[i][j+1]); /* stencil */

dT = T – grid[i][j]; /* local variation */

new_grid[i][j] = T;

if( dTmax < fabs(dT) )

dTmax = fabs(dT); /* max variation in this iteration */

}

}

if( dTmax < epsilon ) /* is the precision reached good enough ? */

finished = 1;

else

{

for( k=0; k<N+2; k++ )      /* Prepare for a new iteration*/

for( l=0; l<N+2; l++ )

grid[k][l] = new_grid[k][l];

}

nr_iter++;

} while( finished == 0 );

gettimeofday( &ts_end, NULL ); /* end the timed section */

/* compute the execution time */

time = ts_end.tv_sec + (ts_end.tv_usec / 1000000.0);

time -= ts_st.tv_sec + (ts_st.tv_usec / 1000000.0);

printf(“%d iterations in %.3lf sec\n”, nr_iter,

time ); /* and prints it */

return 0;

}

- A first UPC program (heat_1.upc)

Starting with the skeleton given in the file “heat_1.upc” in the heat_conduction subdirectory, transform the sequential program present in section 3 to UPC. Comments are present in the following skeleton to guide you in this transformation.

- Complete the shared variable declarations

- Complete the if statements to do a sequential execution of

  - initialize()

  - print maximum execution time

- Add synchronization barriers where required

- Complete the affinity parameter in the upc_forall() statements

- Reduce the value of dTmax and times to get the maximum value across all the THREADS

Note: We assume in the following that (N+2)%THREADS = 0, and that (N+2) is divisible by THREADS

- A better memory use (heat_2.upc)

The previous UPC program is copying the complete grid_new[][] to grid[][] at the end of each iteration. This step creates an overhead and can be avoided by flipping the pointers to these two arrays. The grids grid[][] and grid_new[][] will be accessed using ptr[][] or ptr_new[][], respectively.  At the end of an iteration, ptr[][] and ptr_new[][] pointers are swapped and a new iteration begins.

Take the heat_2.upc file and:

- Declare two new arrays of blocked private pointers to shared, having the same block sizes as grid[][] and new_grid[][] – The dimension of each of these two pointer arrays can be only N+2 since each pointer only needs to point to the beginning of each row of grid[][] or new_grid[][].   A single shared pointer can also be declared for use as a temporary storage for the pointer swapping operation

- Initialize these two pointer arrays, ptr[] and ptr_new[]

- Change the stencil computation to use ptr[] and ptr_new[]

- Replace the grid copy step at the end of each iteration by a pointer flipping step

- Remove any unnecessary synchronization barrier

- Performance boost using privatization (heat_3.upc)

It has been shown that the speed of UPC private memory accesses are much faster than accesses to the shared space, even the local shared space.  Thus in this exercise we consider a hand tuning optimization step, which refer to as privatization or treating local shared space as if it were private to avoid address translation overhead.  In the long run, privatization should be accomplished automatically by UPC compilers.  For now, this can be accomplished by casting pointers-to-shared to private pointers, and using the private pointers to access the local shared data.

Starting from heat_3.upc, create a new variant of the UPC program, after privatization step:

- Duplicate the previous pointers, and declare them private. The size of *ptr_priv[] and *new_ptr_priv[] is only the number of “local” rows, not the total number of rows

- Initialize these new private pointers, by pointing to the local rows of grid[][] and new_grid[][]

- Split the original upc_forall( j=1; j<N-1; j++ ) loop into three parts. Thus, the processing of the first row, middle part and last row of the local chunk are separated.  This way, the middle part can take advantage of privatization while guard zones are still handled with ease using shared pointers

- Adapt each of these three computational blocks to use the private pointers when possible

- Add a pointer flipping step for these private pointers at the end of each iteration

- Change the program to accommodate a 128x128 grid.  Compile and run with 1 thread, 4 threads, and 8 threads and sketch the scalability.  You may consider doing this step using the solution provided in the solutions directory.