%N opcal

%D Correct data for atmospheric opacity and flux miscalibration.

%P rjs

%: uv analysis

%B

opcal corrects a visibility dataset for atmospheric opacity

and can also attempt to correct for errors in the flux calibration

scale. This should be the first step in the calibration of visibility

data (i.e. before any other calibration steps). It should be performed

on both the source of interest as well as all calibrators.

opcal works by computing the brightness and opacity of the atmosphere.

This is estimated from a model of the atmosphere, given the observing

frequency and elevation, as well as meteorological data.

Opacity correction is probably not warranted if the

fluctuations in opacity are small and the calibrator is quite close

to the program source. However opacity correction will not be

damaging in these instances (it would be just an unnecessary extra

step). At frequencies above 15 GHz, opacity correction is generally

recommended.

To correct for atmospheric opacity, opcal scales up the measured

visibility data to account for this attenuation. It also scales the

system temperature data to an ``above atmosphere'' value.

opcal can also estimate and correct for a miscalibration in the flux

calibration scale. The miscalibration might arise from incorrect

value assumed for the on-line calibrator source (noise diode), or

an incorrect conversion to system temperature. opcal works by

comparing its computed estimates of the sky brightness with the

measured values of system temperature. In estimating a miscalibration

factor, it assumes that the measured system temperature is a result of

the the atmosphere and CMB, plus a constant (i.e. contributions to

system temperature from the receivers, spillover, astronomical source,

etc, are assumed constant). A scale factor to correct the measured

system temperature is then computed. Using this procedure should only

be attempted when the observation samples an appreciable range of

elevations.

NOTE: The model sky brightnesses are just that - models, not reality.

They will probably be quite inaccurate in cloudy weather, and

very inaccurate in rainy weather. Do not use this except in clear

weather.

NOTE: This procedure to estimate flux calibration factor is not

necessarily correct. The system temperature scale may differ from

with the model data because the system efficiency used in the

conversion process (the telescope Jy/Kelvin) was wrong, and the

system temperature scale may have been adjusted to account for this.

%A vis

The name of the input uv data set. No default.

%A select

Normal Miriad uv data selection. See the help on "select" for

more information. This selection criteria is used in checking

which data are to be used in determining the flux scale calibration

(i.e. mode=flux or mode=both). All data (regardless of the select

keyword) is copied to the output file.

%A out

The name of the output uv data set. No default.

The output dataset contains extra uv variables, which might be

instructive to examine. These are

tsky      Expected sky brightness, in Kelvins

trans     Expected atmospheric transmissivity (i.e. a fraction,

with 1 indicating a transparent atmosphere).

airtemp   Measured air temperature, in celsius.

pressmb   Measured air pressure, in millibars.

relhumid  Measured relative humidity, as a percent.

antel     Antenna elevation, in degrees.

airmass   Airmass - cosec(antel)

%A mdata

Input text file giving the ATCA meteorology data. No default.

For more help on getting the data appropriate to your

observation, see the help on "weatherdata".

%A mode

This determines the calibration operations to perform. Possible

values are ``opacity'', ``flux'' or ``both'' or "neither". The

default is to perform opacity correciton only.