%N uvdecor

%D Apply decorrelation correction to visibility file

%P pjt

%: uv analysis

%B

UVDECOR acts similarly to UVCAT but applies a correction factor

(>1) to the visibility amplitudes to account for the amplitude loss

due to decorrelation.  The integration time is divided by the

square of the correction factor to reflect the increased noise.

The correction factor is normally derived from the CARMAphase monitor

data, which is scaled to the appropriate frequency, baseline, and elevation

using a simple prescription for 3D Kolmogorov turbulence.  This task

should be used with caution, as the correction is only valid over

long (>10 min) averaging intervals, and the assumption of 3D

turbulence is unlikely to apply on long (>100m) baselines.

Another optional correction factor should be derived, again with caution,

for CARMA data preceding 26-aug-2007 where decorrelation was present

down to about 8000 ns in the delaylength difference between two

antennas.

%A vis

The names of the input uv data sets. Multiple names can be given,

separated by commas. At least one name must be given.

%A select

The normal uv selection commands. One unusual aspect of this is that

the "window" subcommand can be used to select which windows are

copied to the output file (normally the "window" only has an

effect for velocity line type). The default is to copy everything.

%A rmpmax

1 or 2 parameters.  First, maximum acceptable value for rmspath

in microns.  Default=2000.  Second, type of replacement for bad

values: "zero" to set rmspath=0 (no correction), "extrap" to use

previous value.  Default="zero".

%A cormax

Maximum allowed correction factor for the visibility amplitude.

If correction is not prevented by rmpmax, it can still be prevented

at this stage.  Default=100.

%A stokes

If a value is given, uvdecor converts the input into the required

polarizations before writing to the output. Default is to copy

across the polarizations present in the input files.

%A delaymax

If used, this will be the delay difference length (in nanosecs) at

which the decorrellation would have been 0.

If 0 is set, this option is not used.

A good value for CARMA data prior to 26-nov-2007 is about 8500 (TBA).

%A options

This gives extra processing options. Several options can be given,

each separated by commas. They may be abbreivated to the minimum

needed to avoid ambiguity. Possible options are:

'nocal'       Do not apply the gains file. By default, UVDECOR

applies the gains file in copying the data.

'nopass'      Do not apply bandpass corrections. By default, UVDECOR

applies bandpass corrections if possible.

'nopol'       Do not apply polarization correction. By

default UVDECOR corrects polarizations, if possible.

'nowide'      Do not copy across wide-band channels.

'nochannel'   Do not copy across spectral channels.

'unflagged'   Copy only those records where there are some

unflagged visibilites.

%A out

The name of the output uv data set. No default.