%N cgdisp

%D Displays and overlays images on a PGPLOT device

%P nebk

%: plotting

%B

CGDISP displays/overlays images via contour plots, pixel map

representations, vectors and scaled boxes on a PGPLOT device.

Upto 3 contour plots, one pixel map, one vector plot and one box

display may be overlaid in multi-panel plots of multi-channel

images.  In addition overlay locations (plotted as crosses,

boxes, circles, lines or see-through) may be specified from an

ascii text file.

Manipulation of the device colour lookup table is available

when you display with a pixel map representation (formerly

called a "grey scale")

%A in

You may input up to seven images.  Up to three of these can

be displayed via contour plots and one can be displayed via a

colour pixel map representation.  One vector amplitude image and

one vector position angle image (degrees; positive N -> E) can

together be used to display a vector map (e.g. polarization

vectors).  One image can be displayed as small scaled boxes (see

below) and one image may be used as a mask.

The "box" image is displayed by drawing little boxes (solid and

hollow for positive and negative pixels) at the location of each

selected pixel.  The size of the box scales with the value of

the pixel.  This is a useful way to display rotation measure

images for example. The mask image blanking mask is logically

ANDed to all the other image masks before they are displayed.

The mask image is not displayed.

If more than one image is specified, they must have identical

first and second dimensions.  However, you can overlay

combinations of 2-D with 3-D images (e.g. multi-channel images

with a continuum image) provided all the 3-D images have the

same third dimension.  These images can be input in any order

(see TYPE).  Wild card expansion is supported.  No default.

%A type

Specifies the type of each image, respectively, listed in the IN

keyword. Minimum match is supported (note that "pixel" was

formerly "grey" [which is still supported]).   Choose from:

"contour"   (contour;            up to 3 of these)

"pixel"     (pixel map;          up to 1 of these)

"amplitude" (vector amplitude;   up to 1 of these)

"angle"     (vector pos'n angle; up to 1 of these)

"box"       (box;                up to 1 of these)

"mask"      (mask;               up to 1 of these)

You can't give one of "amplitude" or "angle" without the other.

Default is "pixel" for one image, "contour" if more than one.

%A region

Region of interest.  Choose only one spatial region (bounding

box only supported), but as many spectral regions (i.e. multiple

IMAGE specifications) as you like.   Each channel (or group of

channels; see CHAN below) is drawn on a new sub-plot.

NOTE: the region specification applies equally to all the

input images.

Default is full image

%A xybin

Upto 4 values.  These give the spatial increment and binning

size in pixels for the x and y axes to be applied to the

selected region.  If the binning size is not unity, it must

equal the increment.  For example, to bin up the image by 4

pixels in the x direction and to pick out every third pixel in

the y direction, set XYBIN=4,4,3,1

Defaults are 1,XYBIN(1),XYBIN(1),XYBIN(3)

%A chan

2 values. The first is the channel increment to step through the

image in, the second is the number of channels to average, for

each sub-plot.  Thus CHAN=5,3  would average groups of 3

channels together, starting 5 channels apart such as: 1:3, 6:8,

11:13 ...  The channels available are those designated by the

REGION keyword.  A new group of channels (sub-plot) is started

if there is a discontinuity in the REGION selected channels

(such as IMAGE(10,20),IMAGE(22,30).  The combination of REGION

and CHAN determines how many sub-plots there will be.

In the case that you have input some combination of 2-D and 3-D

images, CHAN refers to the 3-D image(s). Note that a channel

is defined to be a pixel on the third axis of a cube, regardless

of the cube's order (xyv or vxy say).

Defaults are 1,1

%A slev

Up to 3 pairs of values, one for contour image. First value is

the type of contour level scale factor.  "p" for percentage and

"a" for absolute.   Second value is the factor to scale LEVS by.

Thus, SLEV=p,1  would contour levels at LEVS * 1% of the image

peak intensity.  Similarly, SLEV=a,1.4e-2 would contour levels

at LEVS * 1.4E-2

Default is no additional scaling of LEVS (i.e., "a",1.0)

%A levs1

The levels to contour for the first specified contour image are

LEVS1 times SLEV (either percentage of the image peak or

absolute).

Defaults try to choose something vaguely useful.

%A levs2

Levels for the second contour image.

%A levs3

Levels for the third contour image.

%A cols1

PGPLOT colours for LEVS1 contours.  If one value is given it is

used for all contours.  PGPLOT colour indices are

0: background colour (black or white)

1: foreground colour (white or black)

2: red           3: green           4: blue

5: cyan          6: magenta         7: yellow

8: orange        9: lime           10: spring green

11: azure        12: violet         13: rose

14: dark grey    15: light grey

%A cols2

Colours for the second contour image.  Defaults to those for the

first image.

%A cols3

Colours for the third contour image.  Defaults to those for the

first image.

%A range

Up to N groups of four values, one group per sub-plot, where N

is the maximum number of channels allowed by Miriad (currently

32768).  The four values are

- MINimum image intensity to display

- MAXimum image intensity to display

- Transfer function type:

lin: linear

sqr: square root

log: logarithmic

heq: histogram equalization

- Colour lookup table:

1: b&w

2: rainbow

3: linear pseudo colour

4: floating zero colour contours

5: fixed zero colour contours

6: rgb

7: background

8: heat

9: absolute b&w

Negate the table number to reverse the lookup table.

The transfer function changes available with OPTIONS=FIDDLE

are in addition to (on top of) the selections here, but the

colour lookup table selections will replace those selected here.

All subplots following the last one with a specified "range"

will use the "range" settings from the previous subplot. In

this way, one group of settings can be applied to all the

subplots if desired.  The multiple subplot capability is useful

if you have used IMCAT to put unlike images into planes of

a cube and you wish to display them together.

Default is linear between the image minimum and maximum with

a b&w lookup table.   You can default the intensity range with

zeros, viz. "range=0,0,log,-2" say.

%A vecfac

3 or 4 values.  A scale factor to multiply the vector image

lengths (or box image widths) by, the x and y increments (in

pixels) across the image at which to plot the vectors (or

boxes), and optionally the length of the scale-bar vector

(unset for no scale-bar). If you have set non unit values of

XYBIN, the increments here refer to the binned pixels.  When

VECFAC(1)=1, the vectors (boxes) are scaled so that the maximum

amplitude (width) takes 1/20 of the (sub)plot size.

The scale-bar gives a graphical representation of the vector

lengths, which makes vector plots easier to interpret.  The

scale-bar is drawn in the corner specified by the BEAMTYP key

(defaulting to bottom-left if BEAMTYP is not specified). If

VECFAC(4)=0, the scale bar is drawn the length of the longest

vector; you can find out what this is using OPTIONS=FULL. For a

fractional polarization vector map, setting VECFAC(4)=1

corresponds to 100 per cent polarization. If VECFAC(1) >> 1,

this will give a very long vector. For polarization intensity

images, VECFAC(4) is specified in flux density.

Defaults are 1.0, 2, VECFAC(2)

Default is not to draw a scale-bar.

%A boxfac

3 values.  A scale factor to multiply the box image widths by,

and the x and y increments (in pixels) across the image at which

to plot the boxes).  If have set non unit values of XYBIN, the

increments here refer to the binned pixels.  When BOXFAC(1)=1,

the boxes are scaled so that there is a little bit of space

between adjacent boxes.

Defaults are 1.0, 2, BOXFAC(2)

%A device

The PGPLOT plot device, such as plot.plt/ps

No default.

%A nxy

Number of sub-plots in the x and y directions on the page.

Defaults choose something depending on your telescope.

%A labtyp

Up to 2 values.  The spatial label type of the x and y axes.

Minimum match is active.  Select from:

"hms"       the label is in H M S.S (e.g. for RA)

"dms"       the label is in D M S.S (e.g. for DEC)

"arcsec"    the label is in arcsecond offsets

"arcmin"    the label is in arcminute offsets

"arcmas"    the label is in milli-arcsec offsets

"absdeg"    the label is in degrees

"reldeg"    the label is in degree offsets

The above assume the pixel increment is in radians.

"abspix"    the label is in pixels

"relpix"    the label is in pixel offsets

"abskms"    the label is in km/s

"relkms"    the label is in km/s offsets

"absghz"    the label is in GHz

"relghz"    the label is in GHz offsets

"absnat"    the label is in natural coordinates as defined by

the header.

"relnat"    the label is in offset natural coordinates

"none"      no label and no numbers or ticks on the axis

All offsets are from the reference pixel.

Defaults are "relpix", LABTYP(1)   except if LABTYP(1)="hms"

when LABTYP(2) defaults to "dms" (to give RA and DEC)

%A beamtyp

Up to 6 values. Set if you want a small polygon to be drawn to

represent the beam FWHM. Setting beamtyp to "b,l" is sufficient to

draw a solid beam; "b,l,4" will result in a cross-hatched

beam.  Use 'n' if you don't want a beam at all.

The six parameters are:

- Beam vertical positioning: can be "t" (top), or "b" (bottom). No

default.

- Beam horizontal positioning: can be "l" (left), or "r"

(right). Default "l"

The next four parameters apply only to the first image specified

with the "in" keyword.  If there are multiple, different beams to

draw (for example, if different uv data were used to produce

images with different beam shapes), all subsequent beams are drawn

as open polygons.

- Hatching style:

1    solid (default)

2    outline

3    hatched

4    cross-hatched

- Hatching angle (default 45 degrees).

- Hatching line separation (default 1).

- Line-width for outlines, hatching and cross-hatching (default 1)

%A options

Task enrichment options. Minimum match of all keywords is

active.

"abut" means don't leave any white space between subplots.  The

default is to leave a little bit between subplots, and

OPTIONS=GAPS leaves a lot of space and labels eacg subplot

separately.

"beamAB", where "A" is one of "b" or "t" and

"B" is one of "l" or "r"

means draw the beam FWHM on the plot in the corner indicated

by the "AB" location. This option is deprecated: use the

keyword "beamtyp" instead.

"blacklab" means that, if the device is white-background, draw

the axis labels in black. Default is red.

"conlabel" means label the contour values on the actual

contours.  The PGPLOT routine that does this is not very

bright.  You will probably get too many labels.  If you bin

the image up with keyword XYBIN, say, by a factor of 2, you

will get about 1/2 as many labels.  If desperate use the

overlay facility (keyword OLAY) to manually label contours.

"fiddle" means enter a routine to allow you to interactively

change the display lookup table.  You can cycle through a

variety of colour lookup tables, as well as alter a linear

transfer function by the cursor location, or by selecting

predefined transfer functions (linear, square root,

logarithmic, histogram equalization)

For hard copy devices (e.g. postscript), a keyboard driven

fiddle is offered; you can cycle through different colour

tables and invoke the predefined transfer functions, but the

linear fiddler is not available.   Note that if you are using

"cgdisp" from a script, so that interactive fiddling is not

appropriate, you can use the "range" keyword to specify the

transfer function and colour lookup tables.

"full" means do full plot annotation with contour levels, pixel

displa range, file names, reference values, etc.  Otherwise

more room for the plot is available.

"gaps" means leave large gaps between subplots and individually

label the axes of each subplot. By default, the subplots will

have a small amount of white space between each subplot and

they will only be labelled around the borders of the full

page.  See also OPTIONS=ABUT to eliminate the small amount of

white space.

"grid" means draw a coordinate grid on the plot rather than just

ticks

"mirror" causes all specified contour levels for all images

to be multiplied by -1 and added to the list of contours

"nodistort" means that angularly-defined overlays do not distort

with the coordinate grid.  If you are displaying a large area

of the sky, such that the non-linearities in the coordinate

system can be seen, then by default, the overlays (keyword

OLAY) will distort with the coordinate grid if you are using

angular units for the overlay locations and half sizes.  Thus

star overlays will rotate and stretch, circles will distort

similarly.  Overlays given in non-angular units will always be

undistorted.

"noepoch" means don't write the epoch value into the axis labels

"noerase" means don't erase a rectangle into which the "3-axis"

values and the overlay ID strings are written.

"nofirst" means don't write the first x-axis label on any

subplots except for the left-most one. This may avoid label

overwrite.

"corner" means only write labels in the lower left corner of any

subplot

"relax" means issue warnings when image axis descriptors are

inconsistent (e.g. different pixel increments) instead

of a fatal error.  Use at your peril.

"rot90" rotates vectors by an extra 90 degrees.  Useful

to convert E-vectors into B-vectors

"signs"  Normally, when plotting vectors, CGDISP assumes that

North is up and East to the left.  If OPTIONS=SIGNS, then

it assumes that E and N are in the direction of increasing

X and Y.

"single" means that when you have selected OPTIONS=FIDDLE and

you have more than one subplot per page, activate the fiddle

option after each subplot rather than the default, which is

to fiddle only at the end.  In the latter case, the histogram

equalization, if invoked, will have been computed with the

image in the last subplot only.

"solneg1" means make negative contours solid and positive

contours dashed for the first contour image. The default,

and usual convention is the reverse.

"solneg2" SOLNEG1 for the second contour image.

"solneg3" SOLNEG1 for the third contour image.

"trlab" means label the top and right axes as well as the bottom

and left ones.  This can be useful when non-linear coordinate

variation across the field makes the ticks misaligned

"unequal" means draw plots with unequal scales in x and y

so that the plot surface is maximally filled.  The default

is for equal scales in x and y.

"wedge" means that if you are drawing a pixel map, also draw

and label a wedge to the right of the plot, showing the map

of intensity to colour.

"3pixel" means label each sub-plot with the pixel value of

the third axis.

"3value" means label each sub-plot with the appropriate

value of the third axis (e.g. velocity or frequency for an

xyv ordered cube, position for a vxy ordered cube).

Both "3pixel" and "3value" can appear, and both will be

written on the plot.  They are the average values when

the third axis is binned up with CHAN.  If the third axis

is not velocity or frequency, the units type for "3VALUE"

will be chosen to be the complement of any like axis in the

first 2. E.g. the cube is in vxy order and

LABTYP=ABSKMS,ARCSEC the units for the "3VALUE" label will be

arcsec.  If LABTYP=ABSKMS,HMS the "3VALUE" label will be DMS

(if the third [y] axis is declination).  See also keyword

"3format" where you can input the format for the "3value"

labelling.

%A 3format

If you ask for "3value" labelling, this keyword allows you

specify the FORTRAN format of the labelling.  I have given

up trying to invent a decent algorithm to choose this. Examples

are "1pe12.6", or "f5.2" etc   If you leave this blank cgdisp

will try something that you probably won't like.

%A lines

Up to 6 values.  The line widths for the axes, each contour

image (in the order of TYPE), the vector image, and any

overlays.  If there are less than 3 contour images or no vector

image, the vector image/overlay line widths shift left.

Line widths must be integers.

Defaults are 1,1,1,1,1,1

%A break

Up to 3 values. The intensity levels for the break between

solid and dashed contours for each contour image.

Defaults are 0.0,0.0,0.0

%A csize

Up to 4 values.  Character sizes in units of the PGPLOT default

(which is ~ 1/40 of the view surface height) for the plot axis

labels, the velocity/channel label, the overlay ID string

(if option "write" in OLAY used) label, and the contour

value labels (see options=conlab).

Defaults try to choose something sensible.  Use 0.0 to default

any particular value. E.g., 1.4, 0, 0, 0.5

%A scale

Up to 2 values.  Scales in natural axis units/mm with which to

plot in the x and y directions.  For example, if the increments

per pixel are in radians, then this number would be radians/mm

(note that for RA axes you give radians on the sky per mm).

Although this choice of unit may be cumbersome, it makes no

assumptions about the axis type, so is more flexible.   If you

also chose OPTIONS=EQUAL then one of your scales, if you set

both and differently, would be over-ruled.  If you give only

one value, the second defaults to that.

Defaults choose scales to fill the page optimally. To default

the first but the second, use 0.0,scale(2)

%A olay

The name of a file containing a list of overlay descriptions.

Wild card expansion is active and the default is no overlays.

Miriad task CGCURS OPTIONS=CURSOR,LOG,CGDISP  can be used to

make an overlay file.

Entries in the overlay file can be white space or comma

delimitered or both.  All lines beginning with # are ignored.

**** DO NOT USE TABS ****

Double quotes " are used below to indicate a string.  The "

should not be put in the file.   For all the string parameters

discussed below, you can abbreviate them with minimum match.

Each line describes an overlay and should be as follows:

##### The first 5 parameters in each line must be

1      2       3     4    5

--------------------------------

OFIG  XOTYPE  YOTYPE  ID  WRITE

here

OFIG is the type of overlay; choose from

"sym"     pgplot symbol number (give centre, symbol, and size)

"star"    star (i.e. cross; give centre and half-sizes)

"box"     box (give centre and half-sizes)

"line"    line segment (give ends)

"vector"  directed line segment (give centre, length, and

position angle)

"circle"  filled in circle (give centre and radius)

"ocircle" open circle (give centre and radius)

"ellipse" filled-in ellipse (give centre, half axes and p.a.)

"oellipse open ellipse (give centre, half axes and p.a.)

"clear"   nothing, so you can write the overlay ID string (see

below) without the overlay

Also

"colour"  See below.

"lwid"    See below.

"offset"  See below.

XOTYPE and YOTYPE  give the units of the overlay location (and

overlay half-sizes) contained in the file for the x- and y-

directions, respectively.  Choose from:

"hms", "dms", "arcsec", "arcmin", "absdeg", "reldeg", "abspix",

"relpix", "absnat", "relnat", "absghz", "relghz",

"abskms", & "relkms"  as described in the keyword LABTYP.

Note that OTYPE does not depend upon what you specified for

LABTYP.

ID is an identifying overlay string which can be optionally

written on the overlay; it MUST be in the overlay file whether

you write it on the plot or not).  The ID string is written in

the corner for "star" and "box", at the end for "line", and in

the centre for "circle" and "clear".  The underscore character

"_" is treated a special case and is replaced by a blank before

plotting.  In this way, you can write several words as the

overlay ID; you connect them with underscores in the overlay

file, and cgdisp strips them out before plotting.

WRITE is "yes" or "no" to specify if the overlay ID is to be

written in the overlay figure or not.

##### Parameters beyond number 5 depend upon OFIG, XOTYPE, and

YOTYPE

6   7    8   9  10  11  12  13  14  15          OFIG

--------------------------------------   -----------------

X   Y   SY  SS  CS  CE                   sym

X   Y   XS  YS  CS  CE                   star, box

X1  Y1  X2  Y2  CS  CE                   line

X   Y   VL  PA  SS  A1  A1  A3  CS  CE   vector

X   Y   R   CS  CE                       circle,  ocircle

X   Y   R1  R2  PA  CS  CE               ellipse, oellipse

X   Y   CS  CE                           clear

X,Y defines the center of the overlay in the nominated OTYPE

coordinate system (X- and Y-OTYPE can be different).

(X1,Y1) & (X2,Y2) are the end points of the line segment in the

nominated OTYPE (mixed OTYPEs are supported here too).

For %OTYPE = "abspix ", "relpix", "arcsec", "arcmin", "absdeg",

"reldeg", "absghz", "relghz", "abskms", "relkms",

"absnat" & "relnat" X,Y,X1,Y1,X2,Y2 are single

numbers.

For %OTYPE = "hms" or "dms", the X and/or Y location is/are

replaced by three numbers such as HH MM SS.S or DD MM SS.S.

Thus, if XOTYPE = hms and YOTYPE = dms then the file for

OFIG=box, say, should have lines like

HH MM SS.S   DD MM SS.S   XS   YS  CHAN

XS, YS are the overlay half-sizes in the following units:

%OTYPE = "abspix" and "relpix" in pixels

"hms"    and "dms"    in arcseconds

"arcsec"              in arcseconds

"arcmin"              in arcminutes

"absdeg" and "reldeg" in degrees

"absghz" and "relghz" in GHz

"abskms" and "relkms" in km/s

"absnat" and "relnat" in natural coordinates

XS, YS are optional for OFIG="box" and "star".  The defaults

are XS = 2, YS = XS pixels.

CS to CE is the channel range (image planes) on which to put the

overlays.  If you specify only CS than the overlay is put on

that channel.  If CS = 0 the overlays are put on all channels.

In all cases, CS and CE are optional and the default is 0 (all

channels)

SY is the pgplot symbol to use for "sym".

SS is the pgplot character height to use for "sym" and "vector".

May be set to zero for vectors to omit the arrowhead.  Default

is the character height used for overlay string.

VL is the length of the vector in pixels.

PA is the position angle in degrees, positive N -> E.

A1, A2, and A3 are the PGPLOT arrowhead style parameters:

A1 is the fill-style, 1 (default) for filled or anything else

for outline.

A2 is the acute angle of the arrow point, in degrees.  Default

45.0.

A3 is the fraction of the triangular arrowhead that is cut away

from the back.  Default 0.3.

R is the radius of circle overlays in the units given in the

above list according to XOTYPE only.

R1 and R2 are the ellipse major and minor axes half-widths,

both in units according to XOTYPE.

##### OFIG = COLOUR (or COLOR)

A COLOUR directive can be included at any point in the overlay

file in the format

COLOUR   INDEX

where the literal "COLOUR" or "COLOR" (without the quotes)

starts in column 1, followed by the PGPLOT colour index.  This

changes the graphics overlay colour until the next COLOUR

directive is processed.  PGPLOT colour indices are listed above

for the cols1 parameter.  The default colour index is 9.

##### OFIG = LWID

An LWID directive can be included at any point in the overlay

file in the format

LWID   WIDTH

where the literal "LWID" (without the quotes) starts in column

1, followed by the PGPLOT line width in units of 0.005 inch

(about 0.13 mm) and must be an integer in the range 1-201.  This

changes the graphics line width until the next LWID directive is

processed.  The default width is 1.

##### OFIG = OFFSET

An OFFSET directive can be included at any point in the overlay

file in the format

OFFSET   XOFF   YOFF

where the literal "OFFSET" (without the quotes) starts in

column 1, followed by X and Y offsets which are applied to all

succeeding overlay file locations.

X = X + XOFF;   Y = Y + YOFF

These offsets must be in the same units as the %OTYPE of

succeeding directives.  It is intended so that your overlay

locations can be in, say, arcsec relative to some location which

is not the reference pixel of the image (which is what CGDISP

ultimately wants).   You then specify, with the OFFSET

directive, the offsets between the reference pixel of the

contour/pixel map images and the actual reference location of

your overlay locations.

You can have as many OFFSET directive as you like in the file.

All succeeding directives will apply these offsets until new

ones are defined.  If the directive does not appear, naturally

no additional offsets are added.

The OFFSET directive is not applied to ANY position fields in

succeeding directives that have %OTYPEs that are "hms" or "dms".

I am too lazy to code it.