%N qvack

%D Flag after change to observing setup

%P rjs

%: calibration

%B

Qvack is a Miriad task to flag data for a number of integrations

near an observing setup change. For example, this is used to

flag data that is suspect because the instrument had not settled

correctly after the change in the setup. The visibility data are

should to be in time order.

%A vis

Input visibility data file. No default.

%A select

Standard uv data selection. Default is to select all data.

%A interval

This gives the time interval, in minutes, for which the data are

flagged after a setup change (or before a setup change, if its

value is negative).  The default is 0.1, which flags for

the first 6 seconds after a switch.

%A force

The time interval, in minutes, which indicates the amount of time

that should ellapse before forcing a (virtual) setup change.

Thus you can QVACK on data that has distinct scans but no actual

change in observing setup.  Note that this means the first

INTERVAL amount of data is always flagged. The MODE option is

not needed, nor is it used if you set FORCE

Defaults to no forced setup change.

%A options

Extra processing options. Currently only one exists:

anticipate   Normally the interval given by the "interval" keyword

is relative to the time of the first data found with

a new setup. options=anticipate makes it relative

to the time of the last data before a setup change

occurs.

%A mode

This determines what changes in setup cause flagging. Several

values can be given, separated by commas, and minimum match is used.

Possible values are:

frequency    A change in the frequency setup. Do not use this

option if Doppler tracking is used.

source       A change in the source observed.

mosaic       A change in the observing centre of a mosaiced

observation.