%N selfcal

%D Determine self-calibration of calibration gains.

%P mchw

%: calibration, map making

%B

SELFCAL is a MIRIAD task to perform self-calibration of visibility data.

Either phase only or amplitude and phase calibration can be performed.

The input to SELCAL are a visibility data file, and model images.

This program then calculates the visibilities corresponding to the

model, accumulates the statistics needed to determine the antennae

solutions, and then calculates the self-cal solutions.

The output is a calibration file, ready to be applied to the

visibility data.

%A vis

Name of input visibility data file. No default.

%A select

Standard uv data selection criteria.

%A model

Name of the input models. Several models can be given, which can

cover different channel ranges, different pointing and sources, and

different polarizations of the input visibility data. Generally

the model should be derived (by mapping and deconvolution) from the

input visibility file, so that the channels in the model correspond

to channels in the visibility file. Though the maps can be made using

any linetype, generally "channel" linetype will give best results (??).

The units of the model MUST be JY/PIXEL, rather than JY/BEAM, and

should be weighted by the primary beam. The task DEMOS can be used

to extract primary beam weighted models from a mosaiced image. If

no models are given, a point source model is assumed.

NOTE: When you give SELFCAL a model, it will, by default, select the

data associated with this model from the visibility data-set. This

includes selecting the appropriate range of channels, the appropriate

polarisation type and the appropriate pointing (if options=mosaic

is used). If you use a point source model, if it YOUR responsibility

to select the appropriate data. In particular, you may well want

to select appropriate polarisations, channels and sources.

%A clip

Clip level. For models of intensity, any pixels below the clip level

are set to zero. For models of Stokes Q,U,V, or MFS I*alpha models,

any pixels whose absolute value is below the clip level are set

to zero. Default is 0.

%A interval

The length of time, in minutes, of a gain solution. Default is 5,

but use a larger value in cases of poor signal to noise, or

if the atmosphere and instrument is fairly stable.

%A options

This gives several processing options. Possible values are:

amplitude  Perform amplitude and phase self-cal.

phase      Perform phase only self-cal.

smooth     Determine the solutions in such a way that they are

smooth with time.

polarized  The source is polarised. By default the source is

assumed to be unpolarised. For a polarized source,

SELFCAL cannot perform polarization conversion. That

is, if the model is of a particular polarization, then

the visibility file should contain that sort of

polarization. For example, if the model is Stokes-Q,

then the visibility file should contain Stokes-Q.

mfs        This is used if there is a single plane in the input

model, which is assumed to represent the image at all

frequencies. This should also be used if the model has

been derived from MFCLEAN.

relax      Relax the convergence criteria. This is useful when

selfcal'ing with a very poor model.

apriori    This is used if there is no input model, and the

source in the visibility data is either a planet,

or a standard calibrator. This causes the model data

to be scaled by the known flux of the source. For a

planet, this flux will be a function of baseline. If

the source is a point source, the ``apriori'' option

is only useful if the ``amplitude'' and ``noscale''

option are being used. For a planet, this option

should also be used for a phase selfcal, to get the

correct weighting of the different baselines in the

solution.

noscale    Do not scale the gains. By default the gains are scaled

so that the rms gain amplitude is 1. Generally this

option should be used with the apriori option.

It must be used if selfcal is being used to determine

Jy/K, and should also be used if the model is believed

to have the correct scale.

mosaic     This causes SELFCAL to select only those visibilities

whose observing center is within plus or minus three

pixels of the model pointing center. This is needed

if there are multiple pointings or multiple sources in

the input uv file. By default no observing center

selection is performed.

Note that "amplitude" and "phase" are mutually exclusive.

The default is options=phase.

%A minants

Data at a given solution interval is deleted  if there are fewer than

MinAnts antennae operative during the solution interval. The default

is 3 for options=phase and 4 for options=amplitude.

%A refant

This sets the reference antenna, which is given a phase angle of zero.

The default, for a given solution interval, is the antennae with the

greatest weight.

%A flux

If MODEL is blank, then the flux (Jy) of a point source model can

be specified here. Also used as the default flux for the apriori

option. ** WARNING does not appear right, as the catalog is used **

The default is 1 (assuming the model parameter is not given)

%A offset

This gives the offset in arcseconds of a point source model (the

offset is positive to the north and to the east). This parameter is

used if the MODEL parameter is blank. The default is 0,0. The

amplitude of the point source is chosen so that flux in the model

is the same as the visibility flux.

%A line

The visibility linetype to use, in the standard form, viz:

type,nchan,start,width,step

Generally if there is an input model, this parameter defaults to the

linetype parameters used to construct the map. If you wish to override

this, or if the info is not in the header, or if you are using

a point source model, this parameter can be useful.

%A out

The output gains file.

The default is to write the gains into the input uvdata file.